SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER ARCHITECTURE COURSE CODE: CSA12

COURSE OUTCOME COVERED

CO1: Analyze the functional units of a computer system including CPU, memory, bus  structures, and I/O components by examining instruction execution cycles, addressing  modes, and performance metrics such as CPI, clock cycles, and benchmarking (BL4) Assessment Tool Weightage: 40%

QUESTIONS:5 Total Marks:25

ANNEXURE A

APPLICATION BASED QUESTIONS

Code of Conduct:

I MUTHUPRIYA R (192421002) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

ANNEXURE A

- A smart city surveillance system processes continuous video feeds from multiple cameras, where large volumes of data are transferred between I/O devices, memory, and CPU. During peak hours, the system experiences lag and delayed response in threat detection. Analyze how the interaction between CPU, memory, and I/O contributes to this issue, and explain how different bus structures (single-bus vs multi-bus) and improvements in instruction execution cycle can enhance system performance.

Analysis

A smart city surveillance system continuously receives video from multiple cameras. The CPU processes video frames, memory stores temporary data, and I/O devices transfer video streams. During peak hours, all components compete for system resources, causing delays.

Interaction of CPU, Memory and I/O

- CPU executes image processing and threat detection algorithms.

- Memory (RAM) stores video frames and intermediate results.

- I/O devices continuously transfer camera data to memory.

When multiple cameras send data simultaneously:

- CPU waits for memory access.

- Memory becomes overloaded with frequent read/write operations.

- I/O devices occupy the system bus for data transfer.

- This creates a bottleneck, increasing response time.

Single Bus Structure

- CPU, memory, and I/O share one common bus.

- Only one transfer occurs at a time.

- Heavy traffic causes bus contention.

- Performance decreases.

Advantages

- Simple design

- Low cost

Disadvantages

- Slow data transfer

- High waiting time

Multi-Bus Structure

- Separate buses connect CPU, memory, and I/O.

- Multiple transfers occur simultaneously.

- Reduces bus congestion.

- Improves throughput.

Advantages

- Faster communication

- Better scalability

- Lower latency

Improving Instruction Execution Cycle

The instruction cycle consists of:

- Fetch

- Decode

- Execute

- Memory Access

- Write Back

Performance can be improved using:

- Pipelining

- Cache memory

- DMA (Direct Memory Access)

- Multi-core processors

- Branch prediction

- Parallel processing

Conclusion

Using a multi-bus architecture, cache memory, DMA, and instruction pipelining significantly reduces lag and improves real-time threat detection.

- A real-time stock trading application running on a processor executes millions of instructions per  second, but users report delays in transaction processing during high load. The system has a high  CPI due to frequent memory access and branch instructions. Examine how the fetch–decode– execute cycle impacts execution time in this scenario, and propose methods to reduce CPI and  improve overall CPU performance using suitable architectural enhancements.

Analysis

The processor executes millions of instructions every second. During high load, delays occur because

the CPU has a high Cycles Per Instruction (CPI).

Fetch–Decode–Execute Cycle

Fetch

- CPU fetches instruction from memory.

Decode

- Control unit interprets the instruction.

Execute

- ALU performs calculations or branching.

Write Back

- Result is stored.

Frequent memory access and branch instructions increase execution cycles.

Causes of High CPI

- Cache misses

- Memory latency

- Frequent conditional branches

- Pipeline stalls

- Branch misprediction

Methods to Reduce CPI

- Increase cache size.

- Use branch prediction.

- Instruction pipelining.

- Superscalar processors.

- Out-of-order execution.

- Prefetching instructions.

- Faster RAM.

CPU Performance Formula

CPU Time = Instruction Count × CPI × Clock Cycle Time

Reducing CPI directly improves processor performance.

Architectural Enhancements

- Multi-level cache (L1, L2, L3)

- SIMD execution

- Multi-core processors

- Speculative execution

- Better branch predictor

Conclusion

Reducing CPI through architectural improvements decreases transaction delay and increases system throughput.

- An embedded system based on an 8085 microprocessor is used in an industrial temperature control  unit, where sensors provide input and actuators respond accordingly. However, incorrect  temperature readings are occasionally processed due to improper use of addressing modes in the  program. Discuss how different addressing modes influence data access, identify possible causes  of such errors, and suggest how the instruction set of 8085 can be effectively used to ensure  accurate and reliable operation.

Analysis

An industrial temperature controller reads sensor values and controls actuators. Incorrect addressing modes may cause wrong temperature readings.

Addressing Modes in 8085

1. Immediate Addressing

Operand is part of instruction.

Example:

MVI A,25H

2. Register Addressing

Data is stored in registers.

Example:

MOV A,B

3. Direct Addressing

Memory address is directly specified.

Example:

LDA 2500H

4. Register Indirect Addressing

Register pair stores memory address.

Example:

MOV A,M

5. Implicit Addressing

Operand is implied.

Example:

CMA

Causes of Errors

- Wrong memory address

- Incorrect register pair

- Sensor data stored in wrong location

- Incorrect pointer values

- Accumulator overwritten accidentally

Effective Use of 8085 Instruction Set

- Use LDA to read sensor values.

- Use STA to store processed results.

- Use CMP for threshold comparison.

- Use conditional jumps (JC, JNC, JZ).

- Validate addresses before access.

Example

LDA 2500H

CPI 50H

JC COOL

Conclusion

Correct addressing modes ensure accurate sensor reading, reliable processing, and safe actuator control

- A computing system uses an 8086 microprocessor with segmented memory for executing multiple  programs simultaneously, but it encounters issues like incorrect data access and stack overflow  errors. Evaluate how segmentation (code, data, and stack segments) works in 8086, analyze how  improper segment handling can lead to such problems, and explain how instruction execution and  addressing modes must be managed to ensure correct program execution.

Analysis

The 8086 uses segmented memory to allow efficient memory management. Incorrect segment handling causes wrong data access and stack overflow.

Segments in 8086

Code Segment (CS)

Stores program instructions.

Data Segment (DS)

Stores variables and data.

Stack Segment (SS)

Stores stack, local variables, return addresses.

Extra Segment (ES)

Additional data storage.

Physical Address Calculation

Physical Address = Segment × 10H + Offset

Example

Segment = 2000H

Offset = 1000H

Physical Address = 21000H

Problems Caused by Incorrect Segments

Wrong Data Access

Incorrect DS points to wrong memory.

Stack Overflow

SP exceeds stack size.

Incorrect Program Execution

Wrong CS causes execution of invalid instructions.

Memory Corruption

Improper ES or DS overwrites data.

Addressing Modes

- Immediate

- Register

- Direct

- Register Indirect

- Based

- Indexed

- Based Indexed

Proper Management

- Initialize CS, DS, SS correctly.

- Maintain stack pointer.

- Use PUSH and POP carefully.

- Avoid invalid offsets.

Conclusion

Proper segmentation and correct addressing ensure accurate program execution, efficient multitasking, and reliable memory management.

5. A data communication system represents packet information in hexadecimal format for ease of  debugging and transmission, but a software engineer mistakenly interprets values, leading to  incorrect data processing. Analyze the importance of number system conversions between  hexadecimal and binary in such systems, explain how errors in conversion can affect instruction  execution, and discuss how efficient CPU design and benchmarking can help detect and minimize  such issues in real-time systems.

Analysis

Packet information is represented in hexadecimal. Incorrect conversion to binary leads to wrong instruction interpretation and data processing.

Importance of Hexadecimal

- Compact representation

- Easy debugging

- Maps directly to binary

- Used in memory addresses and machine instructions

Example Conversion

Hexadecimal:

3AH

Binary:

00111010

Effects of Incorrect Conversion

- Wrong opcode execution

- Invalid memory address

- Packet corruption

- Communication failure

- Incorrect branching

Instruction Execution Impact

CPU fetches binary instructions.

If hexadecimal is converted incorrectly:

- Wrong instruction is fetched.

- Decode stage interprets incorrect opcode.

- Execute stage performs wrong operation.

Efficient CPU Design

- Error detection mechanisms

- Parity checking

- ECC memory

- High-speed ALU

- Pipeline verification

- Cache consistency

Benchmarking

CPU benchmarking measures:

- Execution time

- CPI

- Throughput

- Memory bandwidth

- Instruction rate

Benchmarking helps identify:

- Conversion errors

- Memory bottlenecks

- CPU performance issues

- Real-time processing delays

Conclusion

Accurate hexadecimal-to-binary conversion is essential for correct instruction execution. Efficient CPU architecture and benchmarking improve reliability, detect errors early, and enhance real-time communication system performance.

MARKS ALLOCATION

| Criteria | Understanding of Problem Context (20%) | Application of Concepts (20%) | Problem-Solving Approach (20%) | Accuracy of Solution (20%) | Clarity (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understan ding of Problem Context | 20 | Clearly understands the problem and accurately identifies all requirements | Good understanding with minor gaps | Basic understanding; some requirements unclear | Poor understanding or misinterpretation of the problem |
| Application of Concepts | 30 | Accurately applies appropriate concepts to solve complex problems | Applies relevant concepts correctly with minor errors | Applies basic concepts with limited accuracy | Fails to apply appropriate concepts |
| Problem Solving Approach | 20 | Logical, well structured, and efficient approach | Mostly logical approach with minor gaps | Basic approach with some inconsistencies | Illogical or unclear approach |
| Accuracy of Solution | 20 | Completely correct solution with proper justification | Mostly correct with minor errors | Partially correct solution | Incorrect or incomplete solution |
| Clarity | 10 | Clear, well organized, and properly explained solution | Understandable with minor clarity issues | Limited clarity and explanation | Poorly presented and difficult to understand |